# SAFETY PROCEDURE - ELECTRIC MOTOR INSPECTION

Document No: IKI-SOP-MOT-01

Department: Electrical Maintenance

Purpose: This procedure applies to inspection of Motor M-401 and Motor M-402.

## Safety precautions before motor inspection

Notify the control room and obtain a work permit.

Switch off and lock out the motor at the motor control centre; apply a personal lock and tag (lockout/tagout) in line with IKI-SAF-007.

Verify zero voltage with a tested voltage detector.

Wait for the rotor to stop completely and discharge capacitors.

Wear insulated gloves, safety glasses and arc-rated clothing.

Never remove the coupling guard while the motor is rotating.

## Inspection steps

Check winding insulation resistance with a megger. Insulation resistance below 5 MOhm requires winding drying or rewinding assessment. Check bearing noise and bearing temperature. Inspect the terminal box for loose connections and overheating marks.

| Hazard | Control |
| --- | --- |
| Stored electrical energy | Lockout and verify zero voltage |
| Rotating parts | Keep coupling guard fitted |
| Hot surfaces | Allow motor to cool, wear gloves |